# ELSI User Guide

VERSION: 	1.0

DATE:	01/08/2018

Version History

| Version | Date | Description | Prepared by | Approved by |
| --- | --- | --- | --- | --- |
| V0.1 | 01/04/2017 | Initial draft | Adrien Bouyssi | Nick Screen |
| V0.2 | 20/07/2017 | Revised draft | Adrien Bouyssi | Nick Screen |

Contact

transmission.etys@nationalgrid.com

Please include “ELSI” in the email subject.

Contents

1	Introduction	5

1.1	Overview of document structure	5

1.2	Acronyms used	6

2	An Overview of ELSI	8

2.1	Background on ELSI development	8

2.2	Before running ELSI	11

2.3	Disclaimer	Error! Bookmark not defined.

2.4	Navigating through the tool	12

2.5	Functionalities of ELSI	14

3	ELSI’s calculation engine	17

3.1	High level functionality	17

3.2	Running ELSI in practice	19

4	Key user input data	22

4.1	Time representation	22

4.2	Price zones	25

4.3	Zonal electricity demand	27

4.4	Commodities prices	30

4.5	GB & NWE generation mix	31

4.6	Renewables generation simulation	39

4.7	GB transmission network	41

4.8	NWE interconnectors	48

5	Results from ELSI	51

5.1	Overview of result reporting facility	51

5.2	Simulated unconstrained dispatch results	56

5.3	Simulated re-dispatch results	59

5.4	Optimizer diagnostics	63

Appendix A	Geographical scope covered in ELSI	65

A.1	Map of North West European markets represented in ELSI	65

A.2	Map of the GB transmission system (ETYS 2016)	66

A.3	ELSI zones	67

Appendix B	Reference data sources	70

B.1	UK Future Energy Scenarios (2016/17)	70

B.2	BID3 model	70

B.3	Detailed data sourcing	70

Tables

Table 1	Acronyms used throughout this document	6

Table 2	Mapping of ELSI zones to GB system FLOP zones	68

Table 3	Data sources in public ELSI	70

Figures

Figure 1	Assessing the value of GB network reinforcement options	9

Figure 2	Screenshot of Trust Centre Settings	12

Figure 3	Screenshot of Overview_User sheet	13

Figure 4	Screenshot of Tab Selector menu	14

Figure 5	Schematic process flow of ELSI	16

Figure 32	Process flow of ELSI’s calculation engine	19

Figure 33	Screenshot of Control sheet	20

Figure 6	Screenshot of Variables sheet	23

Figure 7	Screenshot of Overview_Periods sheet	23

Figure 8	Illustrative time period assignments for sample Winter & Summer days	24

Figure 9	Screenshot of the Overview_Master_Set sheet	26

Figure 10	Screenshot of the I_OMPs sheet	27

Figure 11	Daily demand as calculated in the Optimiser_System sheet	27

Figure 12	Screenshot of I_Peak_Demand sheet	28

Figure 13	Screenshot of the I_Load_Duration sheet	29

Figure 14	Screenshot of the I_Commodities sheet	30

Figure 15	Screenshot of the I_Plant_Assumptions sheet	32

Figure 16	Screenshot of the C_SRMC sheet	33

Figure 17	Screenshot of the I_NWE_Capacity sheet	34

Figure 18	Screenshot of the I_GB_Capacity_GG sheet	35

Figure 19	Screenshot of the I_GB_Capacities sheet	35

Figure 20	Screenshot of the I_GB_Capacity_check sheet	36

Figure 21	Screenshot of the I_GB_Bid_Offer sheet	37

Figure 22	Screenshot of the C_BidOffer sheet	38

Figure 23	Screenshot of I_Randoms sheet	39

Figure 24	Screenshot of I_Wind sheet	40

Figure 25	Screenshot of I_Solar sheet	41

Figure 26	Screenshot of I_Network_Static sheet	42

Figure 27	Screenshot of I_Network_Capabilities sheet	44

Figure 28	Screenshot of I_Network_Outages sheet	47

Figure 29	Screenshot of Optimiser_Network sheet	47

Figure 30	Screenshot of I_Network_Links sheet	48

Figure 31	Screenshot of Optimiser_Network sheet	49

Figure 34	Screenshot of the O_Period_System sheet	51

Figure 35	Screenshot of the O_Period_Network sheet	52

Figure 36	Screenshot of the O_Annual_System sheet	53

Figure 37	Screenshot of the O_Annual_Network sheet	53

Figure 38	Screenshot of the O_Annual Summation sheet	54

Figure 39	Screenshot of the O_Multi_Results_Template sheet	55

Figure 40	Screenshot of the O_Multi_Summary sheet	55

Figure 41	Screenshot of the O_Multi_Detail sheet	56

Figure 42	Map of North West European markets represented in ELSI	65

Figure 43	Map of the GB transmission system (ETYS 2016)	66

# Introduction

This User Guide is intended to provide the user with an overview of the Electricity Scenario Illustrator (ELSI). The aim is to aid the user in understanding the individual worksheets included within the model and explain the method by which individual components are calculated.

## Overview of document structure

This User Guide to ELSI is structured as follows:

Section 2 provides an overview of the ELSI tool, explaining at high level:

How it got developed over time,

What it is designed for and how it compares to the reference Network Options Assessment (NOA) tool (BID3),

How it operates internally from a functional and technical perspective, and

How to navigate it from a user perspective.

Section 3 describes at high level:

The optimization engine on which ELSI relies for calculating optimal electricity dispatch, and

The various modes available for running the tool from a functional and user perspectives.

Section 4 examines the key user input data:

Time & price zone representation,

Great British (GB) & North-West European (NWE) electricity demand,

Fuel & carbon prices,

GB & NWE generation mix,

Simulation of renewable outputs across GB & NWE,

GB transmission network & NWE interconnectors capabilities,

Section 5 shows the key results reported by ELSI:

Focusing first on results from the unconstrained dispatch e.g. generation, prices, etc.,

Then, drilling down on results from the constrained dispatch including estimates of Balancing Mechanism costs,

Finally, describing the metrics reported to understand the inner workings of the solver on a particular run.

Key use cases are described in the relevant parts of Section 3 to help users update the data set or run the tool in different modes.

The Appendices set out background information as well as explore a few key elements of the model in more details

Appendix A will present the geographical scope covered by ELSI with a maps of GB transmission boundaries & associated ELSI zones, as well as map NWE markets in scope,

Finally, Appendix B will recap provenance of all key data items in the model.

## Acronyms used

Table 1	Acronyms used throughout this document

| Acronym | Meaning |
| --- | --- |
| ACS | Average Cold Spell |
| BID3 | Power Market Model developed by Poyry |
| BM | Balancing Mechanism |
| CAES | Compressed Air Electricity Storage |
| CCC | Committee on Climate Change |
| CCGT | Combined Cycle Gas Turbine |
| CCS | Carbon Capture & Storage |
| CHP | Combined Heat & Power plant |
| CO2 | Carbon Dioxide |
| CP | Consumer Power |
| DSR | Demand Side Response |
| EISD | Earliest In Service Date |
| ELSI | Electricity Scenario Illustrator |
| ETYS | Electricity Ten Year Statement |
| FAQ | Frequently Asked Questions |
| FES | Future Energy Scenarios |
| GB | Great Britain |
| GG | Gone Green |
| INDO | Initial National Demand Outturn |
| LP | Linear Programme |
| NOA | Network Options Assessment |
| NP | No Progression |
| NWE | North Western Europe |
| OCGT | Open Cycle Gas Turbine |
| OFGEM | Office of Gas & Electricity Markets |
| OMP | Other (i.e. NWE) unconstrained Market Prices |
| RIIO | Revenue = Incentives + Innovation + Outputs |
| ROC | Renewable Obligation Certificate |
| RSPI | Relevant System Planning Information |
| SMP | System Marginal Price |
| SO | System Operator |
| SP | Slow Progression |
| SQSS | Security and Quality of Supply Standard |
| SRMC | Short Run Marginal Cost |
| TEC | Transmission Entry Capacities |
| TO | Transmission Operator |
| VOM | Variable Operating & Maintenance costs |

# An Overview of ELSI

## Background on ELSI development

    - Purpose of ELSI

The Electricity Scenario Illustrator (ELSI) is an Excel model created by National Grid. It contains a simple representation of the Great Britain & North West European (NWE) electricity markets.

ELSI was initially devised as part of the RIIO price control review process in 2010 to demonstrate to network users the consequences of National Grid’s transmission investment plans. It was then used to inform the recommendations of near-term network reinforcements in the Network Options Assessment (NOA) reports. Since 2016, NOA has relied on a different tool (BID3) to model electricity dispatch in GB & NWE markets and simulate the costs of GB transmission network constraints.

    - High level overview of business processes represented in ELSI

The Network Options Assessment (NOA) aims to drive developing an efficient, coordinated and economic system of electricity transmission, consistent with the national electricity transmission system security and quality of supply standard.

Its purpose is to make recommendations to the Transmission Owners (TOs) across Great Britain as to which projects to proceed with to meet the future network requirements as defined in the Electricity Ten Year Statement (ETYS). A key aim is also to recommend to Ofgem which of the projects might be suitable for competition.

An important part of the NOA process is the evaluation of the benefits of transmission reinforcements in terms of forecast reductions in constraint costs across a range of scenarios (which can be set against the costs of the project).  As described above, ELSI was previously used for this stage of the NOA process.

Figure 1 below shows the methodology for evaluating network reinforcement projects.

Figure 1	Assessing the value of GB network reinforcement options

Benefits from GB transmission network reinforcement projects can be assessed from the avoided Balancing Mechanism (BM) costs. To do this, we need to calculate BM costs with (Sensitivity) & without (Base Case) the considered network reinforcement project. Assessment of BM costs in either case requires calculation of costs of re-dispatching generators, storage assets & interconnectors when the constraints are enforced. In practice, this requires modelling of:

Unconstrained electricity dispatch (i.e. no GB transmission constraints), which is akin to day-ahead market,

Constrained electricity dispatch taking into account GB transmission constraints, which is akin to the secured outcome required by National Grid,

Dispatch differences between the unconstrained and constrained runs are resolved in the BM: participants can either:

Offer to increase their output: Offers represent the payment from the system operator to a generator when they are instructed to increase output. Offers are usually higher than SRMC to represents the additional increase in cost, over and above the increase in fuel costs as a consequence of increasing output, except for Hydro, Offshore/Onshore Wind and Wave & Tidal where the offer is equal to zero. As the renewable generator would be generating, and therefore receiving the ROC, the need to be compensated by National Grid is removed. Or

Bid to reduce their output: Bids represent the payment from the generator to the system operator when the generator is instructed to reduce their output. Bids are usually lower than SRMC values to account for a proportion of the fuel cost savings as a consequence of reducing output. The Bid is negative (a payment from National Grid to the generator) for Hydro, Offshore/Onshore Wind and Wave & Tidal and is equal to the value of the ROC. As the renewable generator forgoes the ROC when it is instructed to reduce its output, the model assumes that the system operator would have to pay the generator a value equal to the number of ROCs as a means of compensation.

ELSI performs unconstrained & constrained dispatches of the GB electricity system consecutively and post processes the cost of re-dispatching assets with simplified bids & offers. This is in effect a simplified representation of the Balancing Mechanism, with generators putting forward bids to reduce their forecast (i.e. unconstrained) generation levels as well as offers to increase their forecast generation levels. Note that two runs of ELSI (Base Case & Sensitivity as described above) are required to be able to assess the value of one network reinforcement project.

    - Model development history

ELSI has so far been progressed in the following versions – the later versions include additional functionalities.

Version v1: this is the original version produced to demonstrate to the external stakeholders the consequences of National Grid’s investment plans for RIIO-T1. This version included just two Scottish boundaries B2 and B4, and major SYS boundaries for England and Wales. This version has wind randomisation modelling.

Version v2: this is essentially the same version as v1 but also includes summer boundary outage modelling for each of the 26 samples in ELSI.

Version v3: this version includes probabilistic modelling of the conventional generation.

Version v4.1: this version models all Scottish boundaries B1 B2 B3 B4 B5 and B6.

Version v4.2: includes ETYS 2012 boundaries and reinforcements.

Version v5: Updated version for FES and ETYS 2013.  365 days per year modelled.  Includes more detailed wind and boundary modelling.  Generator SRMCs can also vary by year and scenario.

Version v6: Updated version for FES and ETYS 2014.  More detailed interconnector price modelling.  Embedded wind modelled explicitly.

Version v8:  Updated version for FES 2015, ETYS 2015 and NOA1.  Includes embedded solar modelling.  Forecast load duration curves are used instead of historic data.  Rationalisation of worksheets.

Version v9:  Updated version with the outcome of NOA1 included.

Version 2017-1: Updated to include

A choice of geographical scope: GB-only or full NWE dispatch,

A data set that can be released to the public

## Before running ELSI

Disclaimer

National Grid is releasing this information in good faith and has made reasonable endeavours to ensure accuracy of data and broad alignment with SO constraint modelling, users should not place any reliance on the content of ELSI or any calculations within and/or from using the linear program. ELSI must be considered as illustrative only and no warranty can be or is made as to the accuracy and completeness of ELSI

The following assumptions apply to the content of the ELSI:

- The data provided is illustrative only.

The linear program within the tool is used at the User’s own risk.

The information shown is based on information that was available on the indicated dates of update for each input, meaning that more contemporaneous data may exist at the time of use of the ELSI.

Legal disclaimer: Nothing in this disclaimer limits or excludes National Grid’s liability for (a) death or personal injury caused by our negligence; (b) fraud or fraudulent misrepresentation; or (c) any other liability that cannot be limited or excluded by applicable law. Subject to the above, National Grid shall not be liable to you in tort (including negligence) or otherwise, arising under or in connection with your use of the ELSI for any loss including: (a) loss of profits; (b) loss of sales or business; (c) loss of agreements or contracts; (d) loss of anticipated savings; (e) loss of or damage to goodwill; or (f) any indirect or consequential loss.

Hardware & software requirements

The ELSI tool was originally developed using Microsoft Excel 2010 and tested on Microsoft Excel 2013. It has not been tested on earlier versions of Excel so users are advised to run it on Microsoft Excel 2013 or later versions in order for all functionalities to work.

ELSI is an Excel-based tool with VBA code for calculating the optimal dispatch of the electricity system. As such, in order for ELSI to work properly, macros must be:

Enabled: this can be done by clicking the “Enable Macros” button in the top yellow ribbon on first opening of the spreadsheet, and

Trusted: this can be enabled in the Trust Centre Settings as shown below. The code written in the ELSI tool is open source so users can manually inspect & modify it at their discretion.

Figure 2	Screenshot of Trust Centre Settings

Running ELSI requires at least 2 GB of RAM (more for running several instances of the model in parallel). The ELSI model contains ~50 MB of data (input & outputs) stored in Excel format.

## Navigating through the tool

    - Naming conventions

Sheets in the ELSI workbook are organized in five groups:

Red: Overview e.g. control panel, master sets of power plant types, fuels, price zones, etc.

Light blue: Input data e.g. power plant parameters, fuel prices, network capabilities, etc.

Dark blue: Charts to review input data graphically,

Light grey: Intermediate calculations e.g. SRMC or bids & offers of generators, etc.

Dark grey: Charts to review intermediate calculations graphically,

Green: Calculated inputs for the optimiser e.g. zonal demand, available generation capacity, SRMC of generators, boundary capabilities, etc.

Pink: Outputs from the constrained & unconstrained dispatches e.g. balancing mechanism costs of constrained transmission boundaries

The Overview_User sheet (shown below) recaps all the worksheets in each group with a summary description of their contents. It also indicates contacts information for queries related to this model.

Figure 3	Screenshot of Overview_User sheet

Each of the cells in the worksheets is colour coded to aid the user in identifying the inputs, workings and outputs of the model:

Blue indicates a static input. This is data that can be modified; any changes will update the results produced by the model after the completion of a run,

Grey indicates a value that has been derived from an Excel formula, calculated from the outputs of the optimiser,

Green indicates an input only used for the optimiser,

Pink indicates an output from the optimiser.

Several shades of the same colour are used to alert the user to different e.g. formulas, input category so as to facilitate data interpretation and limit accidental errors e.g. copy/paste.

Name ranges have been defined as a scalable & robust way to interface with the solver used for calculating optimal electricity dispatch. Naming conventions for these named ranges are:

“rngSetxxx” for fundamental information underpinning the model,

“rngIPxxx” for passing input data to the solver,

“rngOPxxx” for results written back into the workbook.

Other named ranges are used for referring to key modelling assumptions e.g. the current year e.g. CYear and do not follow a particular naming convention.

    - Managing visible tabs

On the top left corner of each sheet is a “Tab Selector” button (see screenshot below).

Figure 4	Screenshot of Tab Selector menu

This implements a facility to navigate easily though the workbook by hiding or un-hiding selected groups of sheets following the same grouping convention as the coloured groups above. Pressing a button will toggle its colour from green (visible) to red (hidden) back to green again. The “User” button only shows key screens for a quick model run: Control Panel, Network Capabilities & Links, Multi Detail & Summary output sheets.

## Functionalities of ELSI

    - Scope & limitations

The Electricity Scenario Illustrator is designed to handle:

GB & North West European (NWE) markets, including GB transmission boundaries and interconnectors,

A range of electricity generation & storage technologies,

A simplified representation of the day-ahead market (unconstrained dispatch) & rescheduling after gate closure (constrained dispatch),

Four Future Energy Scenarios (FES) which have been developed by National Grid. These are Gone Green (GG), Slow Progression (SP), No Progression (NP) and Consumer Power (CP). They demonstrate four potential scenarios of how the UK will meet its renewable energy and carbon reduction targets.

Simulations of wind output.

ELSI can be run in two modes:

Either full NWE mode i.e. calculate the optimal electricity dispatch over all NWE markets modelled, or

GB-only mode, where only the GB electricity system is dispatched & imports from/exports to connected NWE markets are based on NWE market prices only.

The main limitations of the ELSI model and their likely impact on results are summarized below:

There are no generator dynamic limitations represented, such as the ramp rate or the minimum stable generation limits which will underestimate the operating costs of the generating units.

The default model represents availabilities of generators, except wind, on a seasonal (not monthly) basis which can underestimate network constraints.

The model assumes ideal curtailment of demand and immediate restoration, which will underestimate demand disruption.

The limited modelling of NWE markets gives a simple modelling of interconnector operation, in particular when it comes to re-scheduling flows. To estimate the variability introduced by these simplifications, two methods of interconnector re-scheduling are available in full NWE dispatch mode.

Network representation assumes ideal capacity sharing across the boundary circuits which overestimates the effectiveness of running certain generators out of merit to alleviate network constraints.

The model assumes all generators are available for curtailment, even the ones connected to distribution networks. This would overestimate the flexibility of generators, as current market and engineering constraints are not taken into account in ELSI.

ELSI models dispatch for four diurnal time periods, when most electricity dispatch tools have an hourly (or sub-hourly) time granularity. This limitation can be overcome by updating the definition of time periods. This would entail significantly longer running times.

For reference, modelling discrepancies between BID3 & ELSI have been reviewed in an audit report so the reader can get a sense for the potential differences between future BID3 results (used for NOA) and the ones from the previously published ELSI model (v6).

    - High level workflow

The workflow used in ELSI is set out in Figure 5 below, starting from user-defined inputs to calculated inputs for the dispatch engine to raw and then post-processed dispatch results. Corresponding worksheets are listed between brackets so readers can use this figure as a map to explore this Excel tool.

Figure 5	Schematic process flow of ELSI

# ELSI’s calculation engine

## High level functionality

ELSI is used to model the electricity market assuming that it is perfectly competitive; i.e. there is perfect information for all parties, sufficient competition so that suppliers contract with the cheapest generation first and that there are no barriers to entry or exit from the market.

In practice, ELSI can run several versions of a dispatch algorithm (described in section 3.1.1 below) to estimate the additional Balancing Mechanism cost attributable to GB transmission constraints. The overarching process flow and its main components are presented in section 3.1.2 below.

    - Dispatch algorithm

The main principles of dispatch engine at the core of ELSI’s calculation engine are listed below:

Minimization of system costs: the dispatch engine’s objective is to find the dispatch minimizing generation (SRMC), storage (through round trip efficiency) & transmission (through losses) costs;

Electricity supply/demand balancing: generators in a price zone define a merit order stack i.e. they are ranked in increasing order of SRMC and contribute up to their available capacity. The balancing point is found by intersecting demand level with the merit order stack. Each day in the twenty year modelling horizon considered is dispatched independently;

Coupling of price zones: transmission capabilities between price zones are used to reduce the overall cost of generating the required amount of electricity in all of the interconnected zones. This coupling can lead to long run times in some cases e.g. large interconnection capacity;

Coupling of intra-day time periods: Storage assets (pumped storage, batteries, Compressed Air Electricity Storage or CAES) are assumed to run a daily cycling pattern i.e. the amount of energy in storage at the beginning of each day does not vary over time. This coupling can lead to long run times in some cases e.g. large storage capacity;

Heuristic to estimate operation of reservoir hydro plant: reservoir hydro power plant are initially scheduled to generate only during peak periods, via a simplified optimisation over the entire year, ignoring transmission constraints. As the daily dispatch progresses, differences between actual and forecast generation are used to adjust the scarcity value of stored water. For instance the value of stored water increases when the storage is emptied faster than planned, and conversely it decreases when storage remains fuller than expected.

    - Overarching process flow for estimating BM costs

The main goal of the ELSI tool is to estimate balancing costs resulting from GB transmission constraints. ELSI runs a three-step process to evaluate these costs:

- Unconstrained dispatch: The first is to look purely at the Short Run Marginal Cost (SRMC) of each GB & NWE zonal fuel type and dispatch available generation from the cheapest, through the more expensive types, until the total level of GB demand is met. At this point, the GB network boundaries are assumed to have infinite capacity. In particular, three outputs from the unconstrained dispatch are used in further steps: NWE zonal prices, NWE interconnector flows and generation levels for all GB generators.

- Constrained dispatch: The second step in the market modelling process is to take the unconstrained dispatch of generation and look at the resulting power flows across the GB transmission boundaries. ELSI will compare these power flows with the actual capability of the GB transmission boundaries and re-dispatch generation where necessary in order to relieve any instances where the power flow is greater than the capability (i.e. where a constraint has occurred). The user can choose between two geographical scopes:

  - Simplified GB-only dispatch: This mode focuses on GB transmission zones (i.e. NWE price zones are not considered). The representation of NWE interconnectors is simplified as pure price takers for both import and export. Due to its simplicity and faster running time, this mode is recommended for testing several boundary configurations once NWE market prices have been generated. Note that for the same reason, BID3 uses a similar GB-only mode to assess the value of network reinforcement options once NWE prices are determined.

  - Full NWE dispatch: This mode runs a full NWE dispatch with the additional GB transmission zones. Interconnectors are also re-dispatched to take into account price changes in NWE markets due to GB transmission constraints.

Three modes are available:

  - Unconstrained dispatch only: This mode would only run the unconstrained dispatch and as a result will not output BM cost estimates. This has not been tested for this release.

  - Full NWE unconstrained & constrained dispatch with fixed NWE interconnection flows: This mode runs full NWE unconstrained & constrained dispatches. Interconnection flows from the unconstrained dispatch are carried over into the constrained dispatch. This mode should only be run in with a full NWE dispatch and has not been tested for this release.

  - Full NWE unconstrained & constrained dispatch with re-dispatched NWE interconnectors: This mode runs full NWE unconstrained & constrained dispatches. Interconnectors are also re-dispatched to take into account price changes in NWE markets due to GB transmission constraints. This is the reference mode, which was tested in both full NWE & GB-only geographical scope.

- Estimate Balancing Mechanism costs: The cost associated with moving away from the most economic dispatch of generation (unconstrained dispatch), to one which ensures the transmission network remains within its limits (constrained dispatch) is known as the constraint cost. It is calculated by applying the Bid and Offer prices to re-scheduled GB generation.

Figure 6 shows a schematic view of the calculation engine used in ELSI and recaps how the geographical scope & mode switches mentioned above are handled. The untested parts have been marked in light red for information.

Figure 6	Process flow of ELSI’s calculation engine

## Running ELSI in practice

The Control worksheet (shown below) provides the model’s navigation tools. It allows the user to set the main assumptions to model a particular view of the future.

Figure 7	Screenshot of Control sheet

The main settings to run ELSI are:

Scenario: allows the user to decide which of the four Future Energy Scenarios to run,

Year: allows the user to decide which of the twenty years in the modelling horizon to run,

Mode: allows the user to decide which of the four modes to run

Unconstrained only,

Unconstrained & constrained with GB transmission constraints and fixed interconnector flows,

Unconstrained & constrained with GB transmission constraints and re-dispatched interconnectors,

Geographical scope: allows the user to run ELSI for either

GB-only (this requires populating the NWE unconstrained market prices in the I_OMPs sheet first), or

Full NWE: no pre-requirements for this mode. Due to the increased modelling scope, the full NWE mode id typically significantly more time consuming to run.

The “Run Multiyear” button will start the calculation engine. Run times for each phase will be reported on the Control sheet when execution is complete. The user can chose to either keep or overwrite existing multi-year results from the O_Multi_Summary & O_Multi_Detail sheets by changing the value in the rngIPMYRetainFlag range in the Control sheet.

    - Use case: run several instances of the tool in parallel

It is possible to run several instances of ELSI in order to gather results faster following the steps below:

- Press and hold the ‘Alt’ key,

- While keeping the ‘Alt’ key pressed, right click on the Excel icon in the task bar and click on the ‘Excel 2013’ option,

- Click ‘Yes’ on the “Do you want to start a new instance of Excel?” message box,

- Open & run a new ELSI file from this instance of Excel.

It is important to note that parallelizing running ELSI will have a significant impact on computing power used: each instance of Excel will use one core and each ELSI model will require up to ~2 GB of RAM.

# Key user input data

The ELSI tool was designed to allow for flexible user input data. As a result, users can update existing data as well as simplify or increase the level of granularity for most data items. This is described in various use cases throughout this section.

## Time representation

ELSI relies on several notions to represent time:

Years: 20 financial years from 2017/18 to 2036/2037. These are used to represent long term evolution of the system (fuel prices, generation mix, network reinforcements).

Month: Each year is broken down into 12 months.

Day: Each year is broken down into 365 days. Leap years are not modelled in ELSI. ELSI dispatches the electricity system one day at a time.

Seasons: Years are broken down into three seasons: Winter, Summer & Spring/Autumn (combined as a single season). Seasons are used to simplify parameter entry e.g. seasonal generator availability. Seasons are mapped to days (and months) in the Variables sheet.

Time periods: The 24 hours in the day are split in four time periods: 1-Peak, 2-Plateau, 3-Pick-up/Drop-off & 4-Night Trough. The mapping between hours of the day and time periods is defined on a monthly basis in the Overview_Periods sheet. Time periods are created to group together the hours with similar GB demand levels.

Time slots: Time slots are the combination of a day and a time period, resulting in 1,460 time slots in each year modelled in ELSI.

The figure below shows the Variables tab, to be edited by Administrators only. In particular, time related data fields are listed below:

Current Year (B3): This is the label of year currently being dispatched.

C Year (B4): This is the ID of year currently being dispatched.

Slots Per Day (B5): Number of Time Slots per day (4).

Days Modelled (B6): Number of days in each financial year (365).

Total Slots (B7): Number of Time Slots in each financial year (1,460).

Final Year (B8): Last year in the modelling horizon (2036).

Total Hours (B20): Number of hours per year (8,760).

Mapping of Day to Season to Month (rows 14:18)

Mapping of Time Slot to Hour to Day to Time Period to Month (rows 21:26)

Number of daily hours for each Time Period in each Month (rows 32:44) – from the Overview_Periods sheet.

Figure 8	Screenshot of Variables sheet

Monthly mapping of Time Periods to half-hours of the day is shown in the Overview_Periods sheet.

Figure 9	Screenshot of Overview_Periods sheet

The only sub-diurnal time granularity used in the model is the Time Period so all inputs (e.g. demand) are pre-processed to this level of granularity.

The graph below shows an approximation of which times of day the 4 periods represent, which will vary slightly between seasons:

Figure 10	Illustrative time period assignments for sample Winter & Summer days

The time periods are calculated off-model using a statistical method to split the 24 hours of a day into 4 periods with the most homogeneous demand levels in each month of the year.

    - Use case: add more years

To study any additional years in ELSI, the easiest way is to replace existing years’ data with new years’ data. Otherwise any new year can be inserted to ELSI by following the steps below:

- Insert the required years in the ranges rngSetFiscalYears & rngSetYears in the Overview_Master_Set sheet. These master set ranges are used throughout the tool to index parameters e.g. peak demand. As such, the user would need to:

- Add rows to the Control sheet,

- Update the last year in the Variables sheet,

- Add columns to the input sheet I_Commodities, for both GB (left) & NWE (right),

- Add rows to the input sheet I_OMPs, for all FES considered,

- Add columns to the input sheet I_NWE_Capacity,

- Add columns to the input sheet I_GB_Capacities & all the I_GB_Capacity_xx for all FES considered,

- Add columns to the input sheet I_GB_Bid_Offer, for ROC value & allocation by power generation technology,

- Add columns to the input sheet I_Peak_Demand, for both demand & periphery adjustment ranges,

- Add rows to the input sheet I_Load_Duration,

- Add columns to the input sheet I_Randoms,

- Optionally add rows to the input sheet I_Wind (if wind load factor data is available),

- Add rows to the input sheet I_Network_Capabilities, for all FES considered,

- Add columns to the intermediate calculation sheet C_SRMC, for both GB (left) & NWE (right),

- Add columns to the intermediate calculation sheet C_BidOffer, for both bids (left) & offers (right),

- Add columns to the result sheets O_Multi_Summary & O_Multi_Detail.

- Double check all the occurrences of the ranges rngSetFiscalYears & rngSetYears in the tool have been updated.

    - Use case: add more time periods

A new period can be inserted to ELSI by following the steps below:

- Insert new time periods in the Overview_Master_Set sheet

- Define the new time periods mapping to half-hours in the Overview_Periods sheet,

- Insert new columns to define the mapping of time periods to days & months of the year in the Variables sheet,

- Insert new columns in the input sheet I_OMPs,

- Insert new columns in the input sheet I_Load_Duration,

- Add columns to the input sheet I_Wind,

- Add new rows with de-rated generation capacity in the Optimiser_System (if a new time period is created as opposed to just re-mapping the existing ones), and new columns for proxy interconnection SRMC & Bid/offer prices,

- Add columns to the result sheets O_Period_System & O_Period_Network,

- Double check all the occurrences of the ranges rngSetTimeSlots, rngSetTimeSlotPeriods & rngSetTimeSlotDays in the tool have been updated.

## Price zones

To be able to resolve flows across international or GB transmission boundaries, ELSI determines electricity dispatch by zone:

In an unconstrained run in the full NWE mode, zones represent the following nine NWE markets: Iceland, Norway, Denmark, Germany, Netherlands, Belgium, France, Spain, and Ireland, on top of GB considered as a single zone. A map on the considered NWE countries is presented in Appendix A.1;

In a constrained run, GB is split according to the transmission system boundaries as published in ETYS. A map of GB transmission constraints from ETYS 2016 is shown in Appendix A.2.

The master list of ELSI zones is present in the Overview_Master_Set sheet (shown below) and used throughout the tool for e.g. demand, generation mix, network capabilities, etc.

Figure 11	Screenshot of the Overview_Master_Set sheet

Prices from the modelled NWE markets are required to run ELSI in GB-only mode. These can be populated in the I_OMPs sheet (show in Figure 12 below) either:

- By running ELSI first in full NWE mode: At the end of the run, ELSI will populate unconstrained NWE market prices from the O_Period_System to the I_OMPs sheet, or

- By populating values manually using a different data source.

Figure 12	Screenshot of the I_OMPs sheet

## Zonal electricity demand

ELSI runs a rolling day dispatch of the electricity system for each financial year considered. On a given day, electricity demand is calculated at the time period and price zone granularity in the Optimiser_System sheet (rows 26:29) as shown in Figure 13 below.

Figure 13	Daily demand as calculated in the Optimiser_System sheet

These demand values are themselves calculated from:

Annual peak demand levels (row 24 in the Optimiser_System sheet), originally coming from the I_Peak_Demand sheet, and

Time period value of demand as a percentage of annual peak, coming from the I_Load_Duration sheet.

    - Zonal annual peak electricity demand

Annual peak electricity demand for all price zones and financial years considered is recapped at the top of the I_Peak_Demand sheet (rows 7:54) as shown in Figure 14 below. The demand value represents the forecast Average Cold Spell (ACS) GB demand of each year, which is the highest demand level.

Figure 14	Screenshot of I_Peak_Demand sheet

Annual electricity peak demand for NWE markets is calculated from:

Future energy scenarios of annual peak electricity demand (rows 56:113), and

Annual adjustment factors (AA7:AU16) to apply a corrective factor across all scenarios. This is used to adjust supply and demand as some NWE markets are only modelled partially e.g. Denmark represents only Jutland (West Denmark & Norway represents only Western Norway. In the current model, these have been turned off as the generation capacities for these markets are aligned with their geographical definitions.

Annual electricity peak demand for GB ELSI zones is calculated from future energy scenarios of annual peak electricity demand (rows 148:561).

The demand value represents the forecast average cold spell (ACS) GB demand of each year, which is the highest demand level in the year. It is evaluated for all ELSI zones and Future Energy Scenarios. A simplifying assumption is made that the zonal distribution of demand at peak is reasonably representative of the year round zonal distribution.

In the simplified GB-only mode, the peak demand for GB zones also includes the full export capacity of interconnectors located in the zone.

    - Zonal load duration curves

The zonal electricity load levels for all four time periods of each day are recapped at the top (rows 5:371) of the I_Load_Duration sheet (see Figure 15 below). These demands are reported as a percentage of the annual peak.

Figure 15	Screenshot of the I_Load_Duration sheet

Load values for NWE zones (I5:AR371) are direct inputs to the model and do not depend on the chosen FES. On the other hand, load values for GB zones are differentiated by FES and future financial year (rows 374:7712).

## Commodities prices

Forward fuel & CO2 prices extending through the end of the modelling horizon (2036/37) for both GB (columns B:W) & NWE (columns Z:AU) power plant are stored in the I_Commodities sheet as shown in Figure 16 below. These prices comprise a variety of fuels used in generating electricity as well as carbon dioxide (CO2) emissions. They are used to calculate the Short Run Marginal Costs (SRMC) of power plant in the C_SRMC sheet.

Note that:

- A Lignite discount (cell F4) and a Distillate multiplier (cell H4) allow the user to define prices for lignite and distillate oil by scaling the reference coal & oil prices. There are no oil or lignite generators in GB across all scenarios considered in this model so GB prices for these fuels have been left blank, and

- Transport adders for gas & CHP (gas with CHP efficiency) from NWE to GB are used to derive NWE gas & CHP prices from GB ones.

Figure 16	Screenshot of the I_Commodities sheet

    - Use case: add a new fuel type

An additional fuel type can be created in ELSI by following the steps below:

- Add a new row in the range rngSetFuelTypes & rngSetFuelTypeCostUnits in the Overview_Master_Set sheet,

- Add new rows in the I_Commodities input sheet, for all FES considered and the summary tables at the top,

- Configure one or several power plant type(s) with the created fuel(s) in the input sheet I_Plant_Assumptions,

- Check that this configuration is correctly picked up in the intermediate calculation sheets C_SRMC & C_BidOffer.

## GB & NWE generation mix

ELSI models power generation units for all GB and NWE zones at an aggregated level. The current model differentiates power plant by fuel type e.g. CHP, nuclear & vintaging by tranche for coal & gas capacity e.g. Gas Base(load), Gas Mid(-merit) & Gas Marg(inal).

    - Operational characteristics of electricity generators

The key characteristics of a power plant in this model are defined in the I_Plant_Assumptions sheet:

Plant type (column B): Type of power plant considered. Names refer to the fuel type as well as sometimes the generator’s vintage e.g. New Nuclear.

Fuel type (column I) & associated emission factor (column D): Main fuel used in generating electricity and CO2 emissions from burning 1MWh of fuel. Emissions are set to zero for CCS power plant as well as renewables.

Efficiency: These represent the efficiency of the transformation of fuel energy to electrical energy for each generator type. These are used in calculating the SRMCs in the C_SRMC sheet.

Seasonal availability (columns E:G): This represents the percentage of time that a generator is available to generate in each of the seasonal periods. It is used within the model to determine the generator’s actual capability i.e. a generator with a capacity of 20MW and 70% availability would have a capability of 14MW. This represents scheduled maintenance and average level of expected outages. In the case of non-dispatchable power generation e.g. wind & solar, load factors by time period are also specified on top of seasonal availability rates (see section 4.6 below).

Flex Flag (column J): This is the generation flexibility flag. The system needs a certain amount of flexible generation to allow the system operator to manage variations in demand. Each of the fuel types is given a category from 1-5:

1 includes Coal, Gas (CCGT & OCGT), Renewables (Hydro Run of River, Hydro Storage and Biomass), Oil and User Defined,

2 includes CHP & CHP New,

3 includes Renewables – Offshore Wind, Onshore Wind, Solar and Wave & Tidal,

4 includes Nuclear & Nuclear New,

5 includes Pumped Storage.

Flex Coef. (column K): This is the flexible coefficient:

A coefficient of ‘1’ will be displayed if the Flex Flag is 1 indicating that the plant is flexible,

A coefficient of ‘0’ will be displayed if the Flex Flag is greater than 1 indicating that the plant is not flexible

Conversion factors: these represent the amount of energy available (in MWh) in a typical unit for the appropriate input fuel e.g. 100 therm of natural gas contain ~3.065 MWh of energy. This used to convert the commodity prices in standard ELSI units (£/MWh) for calculating fuel costs associated with each power plant type in the C_SRMC sheet. All assumptions related to energy density of fuels are at the bottom of the Overview_Master_Set sheet,

Interconnector losses: these represent the losses in transmitting power along interconnector proxies to NWE markets (in the GB-only mode). They should align to their counterparts in the I_Network_Links sheet (in the full NWE mode). These are used to increase the importing capacities of these interconnectors as well as define the price thresholds for dispatching the Import & Float interconnectors (as premium/discount on the relevant unconstrained NWE price).

Figure 17	Screenshot of the I_Plant_Assumptions sheet

Conversion factors for fuels (rows 38:42) are taken from the Overview_Master_Set sheet and represent the amount of energy (in MWh) in one standard unit of the fuel e.g. MWh/tonne of coal. These are associated to generator types (column L) and used to calculate generators’ Short Run Marginal Costs (SRMCs) in the C_SRMC sheet as commodity prices are in £ per standard unit of fuel/CO2.

Figure 18	Screenshot of the C_SRMC sheet

Theoretically, SRMCs represents the cost to the generator of increasing output by one unit (MW) in an hour. In ELSI, plant behaviour is simplified such that every MW of energy from a given generator is generated at the same (SRMC) cost. SRMC is calculated by adding CO2 and fuel costs together. The model bases its unconstrained dispatching decision on the SRMC, dispatching the cheapest generation first to meet demand in a given period. Several types of gas & coal power plant are added in the model to get a finer representation of GB & NWE market prices (i.e. the price of the most expensive generator running).

The SRMCs of non-renewable generators in GB & NWE are calculated at the top of the C_SRMC sheet as shown above. The left part of this sheet (columns B:X) is dedicated to GB generators, the right part (columns AA:AW) to NWE ones. SRMCs are calculated by adding the contributions of fuel costs (rows 43:72) and CO2 costs (rows 79:108). SRMCs of inflexible generators e.g. CHPs can be artificially decreased so that they would only stop generating in cases of extreme oversupply. The User should also note that SRMCs in ELSI must remain positive in order for the optimizer to find a solution.

The SRMCs of renewable generators in GB & NWE are calculated by scaling their ranking order by an arbitrary small number (3 pence) so as to maintain consistency between bids & offers (calculated after optimized dispatch) and SRMC (optimized in dispatch) ranking.

On top of SRMC, artificial zonal merit orders are defined with zonal weightings (row 219 in the Optimiser_System sheet) in order to introduce small differences of zonal prices and therefore speed up convergence of the solver. If the SRMC values are changed, it should be done such as to cause no disruption to zonal weightings.

    - Capacity deployment of GB & NWE electricity generators

A key input to ELSI is the scenarios for capacity deployment of NWE electricity generators out to 2036/37 in the I_NWE_Capacity sheet (rows 7:145) as shown in Figure 19 below. This is calculated by selecting the appropriate scenario from the lower half of this sheet (rows 50:717).

Figure 19	Screenshot of the I_NWE_Capacity sheet

The capacity mix of GB zones is setup in the I_GB_Capacity_GG, I_ GB_Capacity_SP, I_ GB_Capacity_NP & I_ GB_Capacity_CP sheets (one for each scenario). These sheets lay out the capacity (MW) for each current or future GB power plant. The original version of ELSI is setup to report these by:

ELSI plant type: Power plant type used to characterize electricity generators (as above),

ELSI zone: This corresponds to GB transmission zones. A map is available in A.3

Financial year: These are the aggregate Transmission Entry Capacities (TECs) of each of the stations for a particular year. These values represent the total amount of generation (MW) capacity of each particular fuel type in a specific zone. The model does not specify the TEC of each individual generating unit; the values are a collective sum of all the units of a given fuel type within a zone.

Figure 20	Screenshot of the I_GB_Capacity_GG sheet

Capacity deployment in the I_GB_Capacities sheet is calculated by selecting the appropriate scenario from the I_ GB_Capacity_xx sheets. These capacities are then reported in the Optimiser_System sheet (rows 32:62).

Figure 21	Screenshot of the I_GB_Capacities sheet

Note that on top of these power generation capacities, a ‘DSR’ generator is also present in ELSI. In ELSI, the DSR generator represents loss of load, not commercial Demand Side Response. As such, it is only used for ensuring feasibility of the dispatch when demand is larger than generation capacity & for debugging purposes as activation of the DSR generator is reported in the O_Period_System result sheet. The DSR generator is configured with

- The same capacity as demand in the zone in which it belongs, and

- A SRMC of £6,000/MWh from the DSR commodity in the I_Commodities sheet (In ELSI this is the equivalent of load loss).

On top of this, a ‘DSR adder’ setting is present in the Optimiser_System sheet: this represents the capacity (in GW) to be added to the DSR generator and is currently set to 50 GW as in ELSI3. This setting is designed to ensure additional capacity margin and speed up solving in ELSI. It should not be modified.

    - Use case: check GB capacity mix

Before running, ELSI will check that the capacity recorded in the I_GB_Capacities sheet matches the one from the corresponding I_GB_Capacity_xx sheet (depending on the active scenario). An indicator of this has been placed in cell F16 of the Control sheet for the User to review before running the tool.

Mismatches can be due to missing rows in the I_GB_Capacities sheet or errors in the formulas at the top of this sheet. The I_GB_Capacity_check sheet (shown below) allows the User to pinpoint which plant type and zone is missing, by calculating the capacity in each of the I_GB_Capacity_xx sheets for the selected year (in cell C3) and comparing it to the capacities in the current I_GB_Capacities sheet (in column H).

Figure 22	Screenshot of the I_GB_Capacity_check sheet

On top of this, charts allows for visualizing the difference between capacity mixes across different FES.

    - BM participation of GB electricity generators

GB electricity generators can either increase or decrease their output to resolve network constraints. To signal their availability for doing so, they place bids in the BM.

The I_GB_Bid_Offer sheet shown below sets out the bids & offers for GB generators as a percentage of SRMC by fuel type (rows 5:18). On top of this, Renewable Obligation Certificates (ROC) often form the basis of bids of renewable generators. The value of ROCs and the allocation of ROCs to renewable generation by technology is recapped in the middle of the I_GB_Bid_Offer sheet (rows 23:34). As a simplifying assumption, ROCs are modelled as a fixed amount and represents the payment to each renewable generator for every MWh it generates.

Figure 23	Screenshot of the I_GB_Bid_Offer sheet

Data in the I_GB_Bid_Offer sheet is used to derive the bids & offers of various GB generators in the C_BidOffer sheet (shown below). Bids are calculated on the left (columns B:W), while offers are on the right (columns Z:AU). Renewable generators are dealt with separately at the bottom (rows 42:67) to determine the ranking order of negative bids

Figure 24	Screenshot of the C_BidOffer sheet

    - Use case: add a new generator type

Adding a new generator type can be done by following the steps below:

- Add a new generator type in the ranges rngSetPlantTypes & rngSetPlantTypeCategories in the Overview_Master_Set sheet,

- Add a new row in the input sheet I_Plant_Assumptions input sheet, complete with the other characteristics of the new power plant type e.g. availability, efficiency, fuel, etc.

- Add a new rows in the input sheets I_GB_Capacities & its sources I_GB_Capacity_xx for each FES to define the location of the new generator as well as its capacity deployment over the considered modelling horizon. Note that the I_GB_capacity_check can help you pinpoint where discrepancies exist between the I_GB_Capacities & its sources I_GB_Capacity_xx for each FES.

- Add a new rows in the input sheet I_NWE_Capacity, for each considered NWE market where the new generator is located.

- Add new rows to the intermediate calculation sheets C_SRMC (for the fuel cost, carbon cost & summary table at the top) & C_Bidoffer (for the renewables bids & ranking orders as well as the summary tables at the top).

- Add new rows in the Optimiser_System sheet, for total & de-rated generation capacity by time period as well as SRMC. New proxy interconnectors for the GB-only mode should be assigned a connected NWE market as well. Do not forget to extend formulas for proxy interconnector SRMCs for all four periods (by scrolling to the right of the Optimiser_System sheet).

- In the case of a new storage (resp. reservoir hydro) generator type, new rows are to be added to the Optimiser_System sheet in the range rngIPPumpStor (resp. rngIPHydro).

- Add new rows in the result sheet O_Period_System (unconstrained dispatch for GB & NWE generators, constrained dispatch for GB & NWE generators, re-dispatched energy for a GB generator only & BM costs for a GB generator only). In the case of a new storage generator type, new rows are to be added to the O_Period_System sheet for raw constrained output, raw unconstrained output, constrained output, unconstrained output, re-scheduled pumped storage & pump BM correction.

- Add new columns in the result sheet O_Annual_System, new rows to O_Annual_Summation, new rows on the left tables only for the result sheet O_Multi_Results_Template, and new rows to the result sheets O_Multi_Summary & O_Multi_Detail.

## Renewables generation simulation

The value of GB transmission network reinforcements can vary significantly depending on generation from non-dispatchable renewables (especially wind). High wind generation can constrain transmission lines and lead to curtailment costs. ELSI represents wind variability by allowing for randomizing wind output in the I_Randoms sheet (show below).

Figure 25	Screenshot of I_Randoms sheet

Pressing the “Random number generator” button will

Generate a set of random numbers for each day of the years in the modelling horizon considered according to a uniform probability distribution between zero and one, and

Paste them in the bottom table (C14:V378).

The Min, Max & quartiles table at the top (C6:V10) is used to control the numbers output by the random generator:

Min value should always be very close to 0 for all years,

First quartile value should be very close to 25% across all simulated years,

Second & third quartiles values (Q2, Q3) are reported as the difference between two consecutive quartiles so their values should be close to 25% for all years,

Maximum value are calculated as the difference between the maximum annual random number and the third quartile so they should also be close to 25% for all years.

Wind generation is calculated in the I_Wind sheet (shown below) either from straight historical data or using the simulated numbers generated in the I_Randoms sheet using the “Mode” toggle (cell J7).

Figure 26	Screenshot of I_Wind sheet

This worksheet contains:

The mapping between price zone & onshore & offshore wind zones if any (D7:F56),

Ten years of historical data (rows 426:3741) which is used as part of the wind calculation (rows 58:422).

In historical mode, the calculated normalized wind output values are the historical values for the chosen historical year (cell L11). In randomized mode, the random numbers are used to sample from the monthly historical distribution of wind output. For instance, a random value of 75% on January 5th would result in picking the .75 * 31 days (in January) * 10 years ~ 233rd value for the historical series of January wind outputs.

Normalized wind output values from the I_Wind sheet are used to calculate available peak generation by period for onshore & offshore wind in the Optimiser_System sheet.

The I_Solar sheet (shown below) contains a mapping of GB transmission zones to solar regions (top left table) as well as a fixed set of solar load factors by month, daily time period & market (row 48:59). Similarly to wind, these are used to calculate available peak generation by period for solar PV in the Optimiser_System sheet.

Figure 27	Screenshot of I_Solar sheet

Run-of-river hydro capacity is de-rated by the hydro input scalars specified in range O7:R9 of the Optimiser_System sheet. The “Min Hydro” function present in the Optimiser_System sheet is no longer in use in this version of ELSI.

## GB transmission network

A representation of the GB transmission network, with zones and boundaries is required to run a (transmission) constrained dispatch i.e. a dispatch where GB is split by transmission zone instead of being considered as an indivisible market (as is the case in the unconstrained dispatch).

    - Network topology configuration

An overview of the zones and boundary capabilities which make up the network can be found on the I_Nework_Static worksheet (shown below). It stores a mapping of transmission zones to the boundaries around them (or network connectivity table). It shows which boundaries would be impacted as a consequence surplus or deficits of generation in a particular zone:

  - A ‘1’ indicates that a surplus of generation is historically expected in that zone affecting the boundary limit, i.e. an exporting boundary,

  - A ‘-1’ indicates that a deficit of generation is historically expected in that zone affecting the boundary limit, i.e. an importing boundary, or

  - A ‘0’ (or no value) indicates that the zone is neither on the importing or the exporting side of the boundary.

- At the bottom of this sheet, there is a count of number of circuits per boundary. They are used to estimate capacity decrease during a scheduled maintenance outage in the I_Network_Outages sheet.

Figure 28	Screenshot of I_Network_Static sheet

The network connectivity table is passed directly to the optimizer.

    - Use case: add a new GB transmission boundary

A new GB transmission boundary can be added in ELSI by following the steps below:

- First, add new rows to the Overview_Master_Set sheet to define the reference set boundary range used throughout the model,

Then, add zone (FLOP zone mapping is optional) & mapping to GB boundaries as well as the number of circuits across the new boundary in I_Network_Static. Conventions are -1 for importing boundary and 1 for exporting boundary as above,

Add new columns to the I_Network_Capabilities sheet to represent the winter capabilities for the new boundary

Add new rows in the I_Network_Outages sheet to specify the days where outages apply

Update input boundary capabilities range (rngIPSubZnBdLimits) & boundary capabilities in the Optimiser_Network sheet by adding new columns,

Add new rows in results sheets O_Annual_Network, O_Annual Summation, O_Multi_Results_Template, O_Multi_Summary & finally O_Multi_Detail, and

Finally, the User can check that all occurrences of range rngSetGBboundaries have been updated.

    - Use case: add a new GB transmission zone

A new GB transmission zone can be added in ELSI by following the steps below:

- Add a new GB zone in Overview_Master_Set sheet,

Add new rows in I_Peak_Demand (in overview table as well as the ones for each individual FES). In the same sheet, update formulas for calculating full GB peak demand & GB total check.

Add new rows to provide room for generation units in the new GB zones in the I_GB_Capacities sheet as well as its sources I_GB_Capacity_xx,

Add new rows in I_Solar and I_Wind sheets to map the new GB zones to solar & wind zones,

Update interconnection links to GB if required (i.e. if the new zone affects an existing zone with a current or proposed interconnector) in I_Network_Links,

Add new columns & update formulas in Optimiser_System,

Add new rows in O_Period_System for tables related to GB prices, generation, DSR, emissions, etc.

Update number of GB zone x plant type combinations in the Variables sheet,

Add new rows to the other result sheets i.e. O_Annual_System, O_Annual_Summation, O_Multi_Results_Template & O_Multi_Detail.

    - Use case: add a new NWE price zone

A new NWE market / price zone can be added in ELSI by following the steps below:

- Add new rows for defining new markets in the Overview_Master_Set sheet,

Add new rows in I_Peak_Demand (in overview table as well as the ones for each individual FES),

Add new columns and load factors in I_Load_Duration

Add new rows (in overview table as well as the ones for each individual FES) & update capacities in I_NWE_Capacity,

Add new rows to define mapping between new market & wind & solar zones, update the overview table (for charting summary load factors) and input load factors in data table for I_Wind & I_Solar sheets,

If required, add interconnection links in I_Network_Links with their commissioning dates & capacities by FES,

If required, add interconnection links in Optimiser_Network and resize range rngIPLinksData,

Add new columns & update formulas in Optimiser_System,

Add new rows in O_Period_System for tables related to NWE prices, generation, DSR, emissions, etc.

    - Network Capabilities

Different scenarios of GB transmission network development can be assessed using ELSI. The sheets I_Scotland_Capabilities & I_Network_Capabilities (shown below) provide the user with two scenarios of GB network reinforcementfor each future energy scenario considered:

The current level of capabilities for all boundaries (as a lower bound),

A Security and Quality of Supply Standard (SQSS)-compliant scenario (as an upper bound).

Figure 29	Screenshot of I_Network_Capabilities sheet

The network capabilities represent the maximum capacity that can be transferred across each individual boundary, after having taken into account the relevant engineering constraints i.e. pertaining to voltage, stability and thermal constraints. In particular:

The winter value is equal to the capabilities,

The spring/autumn value is 90% of the winter value, and

The summer value is 80% of the winter value

In the public version of ELSI, GB transmission boundary capabilities have not been included as these capabilities are considered confidential information. It is left to the user to populate these cells.

The user is able to input total capabilities on each boundary in each of the seasons, as the year and scenario modelled are calculated by retrieving the values corresponding to the combination of year and FES that are active in the model. This total will be the winter value, with the spring/autumn and summer value calculated as 90% and 80% of winter respectively.

    - Use case: configure capabilities resulting from implementation of network reinforcement projects

      - For all years in the modelling period

The I_Network_Capabilities sheet also (optionally) allows the user to configure individual network reinforcement schemes, so that the boundary capabilities can be built from the bottom up.

First, the commissioning year for each future energy scenario would be required (3000 would mean the project does not get built),

The “User Override” column allows the additional capacity of an individual project to be ‘switched off’ by changing TRUE to FALSE. This means that this additional capacity will no longer be included as part of the boundary capability. The statement in the Active column will be automatically updated so that the two columns match. The model does not allow the user to ‘switch on’ the additional capacity before the year stated in the scenario that is currently been modelled. Therefore, switching the user override to TRUE for a project that is displayed as FALSE in the Active column will not make a difference.

The “Active” column indicates if the reinforcement will be active in the year and scenario that the user has selected to model. TRUE indicates that the reinforcement will be active, FALSE indicates that it can become active in a later year.

The Earliest In Service Date (EISD) column is for information only. It is not picked up in the capability calculations.

The “Reinforcement Package” column lists the details of the reinforcement schemes. The additional capacity that will be created as a consequence of the projects is listed under each of the affected boundaries. Complex formulas can be devised to model dependencies between projects as appropriate.

The “Code” column is used to list the project’s four-letter code.

The years listed under each of the generation scenarios represent the year that each of the projects becomes active, which varies depending on the scenario.

Total capabilities on each boundary in each of the seasons, for the year and scenario modelled are calculated by adding the existing capabilities to the additional i.e. from selected (active) future reinforcements. This total will be the winter value and the spring/autumn and summer value will be calculated as 90% and 80% of winter respectively.

      - For a particular year of interest

The impact of network reinforcement projects in a given year & FES can be studied in ELSI with the following steps:

- Add a ‘Base Case’ estimate of boundary capabilities without the project in input sheet I_Network_Capabilities in rows 102 to 104,

- Add a ‘Reinforced Case’ estimate of boundary capabilities with the project in input sheet I_Network_Capabilities in rows 106 to 108,

- Optionally, the user can add another ‘Reinforced Case’ estimate of boundary capabilities with another project in input sheet I_Network_Capabilities in rows 110 to 112,

- Finally, the User would need to point the boundary capability formulas in Optimiser_Network sheet (rows 31 to 33) to the right boundary capabilities i.e. Base Case, Reinforced Case 1 or 2.

    - Network outages

On top of these boundary capabilities, network outages, modelled in I_Network_Outages sheet (shown below) is an important factor to determine available network capability for the optimiser.

In ELSI, outages reduce the transfer capability at the affected boundary on the day the outage occurs (represented with a “Y” in the outage table). Reduced capabilities should be read in the columns to the left of the table (one for each season).

Figure 30	Screenshot of I_Network_Outages sheet

Finally, network input boundary cap in the Optimiser_Network sheet (shown below) are calculated using both network boundary capabilities & outages. These values are used in the optimizer to decide which boundaries are constrained and by how much.

Figure 31	Screenshot of Optimiser_Network sheet

## NWE interconnectors

    - Full NWE dispatch

NWE markets interconnected with GB are modelled in ELSI to assess the contribution of interconnection to resolving GB transmission constraints. In particular, interconnection reinforcement scenarios are defined in the I_Network_Links sheet (shown below).

Figure 32	Screenshot of I_Network_Links sheet

Modelling of existing and projected interconnectors is similar to the one of GB transmission options, so that interconnection capacity can be calculated across all the projects sharing the same international border:

The interconnector “Reference” column lists the common denomination for projected or built interconnectors,

The commissioning year for each future energy scenario would be required (3000 would mean the project does not get built),

The “User Override” column allows the additional capacity of an individual project to be ‘switched off’ by changing TRUE to FALSE. This means that this additional capacity will no longer be included as part of the boundary capability. The statement in the Active column will be automatically updated so that the two columns match. The model does not allow the user to ‘switch on’ the additional capacity before the year stated in the scenario that is currently been modelled. Therefore, switching the user override to TRUE for a project that is displayed as FALSE in the Active column will not make a difference.

The “Active” column indicates if the reinforcement will be active in the year and scenario that the user has selected to model. TRUE indicates that the reinforcement will be active, FALSE indicates that it can become active in a later year.

The Earliest In Service Date (ESID) column is for information only. It is not picked up in the capability calculations.

Interconnection capacity is recorded in the appropriate columns. Note that this allows for capacity to differ by direction e.g. existing import capacity from Germany to France is 3.2 GW but export capacity from France to Germany is only 2.7 GW.

Reinforced capacity (row 38) is calculated by summing projected with existing capacity for each transfer border. Interconnectors’ reinforcement capacities & losses (row 9) are picked up in the Optimiser_Network sheet (shown below) and passed to the solver.

Figure 33	Screenshot of Optimiser_Network sheet

    - GB-only dispatch

In the simplified mode focusing only on GB transmission zones, interconnectors are modelled as price taking generators. For each NWE market connected to GB, two proxy interconnector generators are created:

One (labelled “Float”) to allow for reducing exports. It is configured with the full interconnector capacity (including losses so that the interconnector rating at the NWE market is the full nameplate capacity) and its SRMC represents a low connected market price. This way, the proxy interconnector generator will generate at full capacity, removing the load equivalent to full exports, when the GB zonal market price is slightly lower than the connected market price (to take interconnection losses into account); and

Another one (labelled “Imp”) to allow imports. It is configured with the full interconnector capacity and its SRMC represents a high connected market price. This way, the proxy interconnector generator will generate at full capacity, removing a further load equivalent to full imports, when the GB zonal market price is slightly higher than the connected market price (to take interconnection losses into account).

    - Use case: add a new interconnector project

Adding a new interconnector in ELSI can be done using the following steps:

- For the GB only mode,

  - Add a new Interconnection fuel type according to process laid out in section 4.4.1,

  - Add two new Interconnection plant types (one for Import, the other one for Float) according to process laid out in section 4.5.5. As a reminder, the interconnector capacity & connected zones (NWE market & ELSI zone) are specified in the I_GB_Capacity_xx sheets (one for each FES),

- For the Full NWE mode,

  - Add two new columns and a new row in the input sheet I_Network_Links for specifying interconnection characteristics e.g. losses, availability, connected zones, capacity and commissioning dates for each FES.

It should be noted that the characteristics of the proxy interconnector in the GB-only mode should be identical to the ones of the same interconnector in the full NWE mode. In particular, this explains the restriction on proxy interconnector availability in the I_Plant_Assumptions input sheet (only annual availability is allowed in the I_Network_Links sheet).

# Results from ELSI

After each run, ELSI produces a large amount of results describing the simulated dispatch of the modelled markets and the re-dispatch of GB connected generators, interconnectors & storage units once GB transmission boundary constraints have been applied. This section will provide a short description of these results, focussing first on outputs from the initial dispatch and then on outputs from the sub-sequent re-dispatch after GB transmission constraints are enforced.

## Overview of result reporting facility

At high level, results are organized in the tool as follows:

The sheet O_Period_System, shown below in Figure 42

- Reports generation & prices for both GB & NWE zones, in the unconstrained & constrained runs;

- Shows BM costs by plant type for generators & storage assets;

- Allows for reviewing demand, loss of load & emissions for both GB & NWE zones, in the unconstrained & constrained runs, and

- Provides debugging information to analyse the solver’s performance.

Figure 34	Screenshot of the O_Period_System sheet

The sheet O_Period_Network, shown below in Figure 43,

- Reports link (e.g. interconnector) flows for both GB & NWE, in the unconstrained & constrained runs and calculates the associated BM costs; and

- Shows GB transmission boundary flows in the unconstrained & constrained runs & calculates estimates of constraint costs from shadow prices & system-wide BM costs.

Figure 35	Screenshot of the O_Period_Network sheet

The sheet O_Annual_System, shown below in Figure 36 reports unconstrained & constrained generation & load factors by technology type as well as prices, emissions & demands on an annual level, picking up summary results from the O_Period_System sheet.

Figure 36	Screenshot of the O_Annual_System sheet

The sheet O_Annual_Network, shown below in Figure 37 presents the main results from links (e.g. flows, time spent importing/exporting) & GB transmission boundaries (flows, quartiles) for both constrained & unconstrained runs. These are in general taken from summary results from the O_Period_Network sheet.

Figure 37	Screenshot of the O_Annual_Network sheet

The sheet O_Annual Summation, shown below in Figure 38 reports a selection of annual results e.g.

Generation for constrained, unconstrained runs, bid/offer volumes;

Demand, DSR & emissions;

GB boundary quartiles & estimated constraint costs.

Figure 38	Screenshot of the O_Annual Summation sheet

The sheet O_Multi_Results_Template, shown below in Figure 39 acts as a temporary placeholder for annual results to be copied over to the O_Multi_Summary (column B) or O_Multi_Detail sheets (column G).

Figure 39	Screenshot of the O_Multi_Results_Template sheet

The sheet O_Multi_Summary, shown below in Figure 40, is a repository for several years of annual results, e.g.

- Estimated boundary constraint value,

- Constrained generation & interconnection flows,

- Zonal price & emissions.

Figure 40	Screenshot of the O_Multi_Summary sheet

The sheet O_Multi_Detail, shown below in Figure 41, is a repository for several years of annual results, e.g.

- Boundary flows, statistics & estimated BM costs,

- Constrained & unconstrained DSR volumes,

- GB generation bid/offer volumes,

- Zonal within-day prices.

Figure 41	Screenshot of the O_Multi_Detail sheet

## Simulated unconstrained dispatch results

Results from the initial dispatch in ELSI can be grouped in four categories:

- Dispatched energy volumes from various power plant types,

- GB boundary flows (even when no transmission capability constraint is enforced).

- Market-level prices resulting from the unconstrained dispatch, and

- Emissions from resulting from the unconstrained dispatch.

    - Unconstrained dispatched energy volumes

Energy is supplied from various sources to meet demand in the modelled NWE markets. ELSI reports unconstrained electricity generation at different levels of granularity for the user to review.

      - Generation from power generators

ELSI reports unconstrained generation from power generators for both GB (by transmission zone) & NWE markets at various levels of time aggregation:

- In the O_Period_System results sheet, unconstrained GB generation (in MW) (incl. proxy interconnectors to GB) for each time period is written in the range rngOPGBUnConMO, and similarly detailed unconstrained NWE generation is written in the range rngOPEUUnConMO, and

- In the O_Annual_System results sheet, annual unconstrained generation by plant type (including GB interconnectors) & zone is written in rows 6 to 57. The left hand side table calculates the annual energy generated, when the right hand side table evaluates the average annual load factor for each of these technologies in each of the markets & zones considered.

      - Generation from reservoir hydro generators

Reservoir hydro (named ‘Hydro Storage’ in ELSI) dispatch is modelled using a special heuristic in ELSI. Its generation volumes aggregated at the time period level are reported with the other generators but are also reported separately in the range rngOPHydroGen in the O_Period_System result sheet. The value of water resulting from the reservoir hydro heuristic is also written in the range rngOPHydroBid of the same sheet.

      - Pumped & released energy from storage assets

Storage assets can both inject (release) energy to and withdraw (pump) energy from the system. ELSI reports unconstrained generation from storage assets with other generators (as above) but it is also setup to report these quantities separately.

In the O_Period_System result sheet, (pumped) storage unconstrained generation (in MW) (both injection as a negative figure & withdrawal as a positive figure) is written in the range rngOPUnConPS. Corresponding unconstrained generation (in MWh) is calculated in rows 3908 to 3979 of the same sheet. It is used to separate annual generation between positive (released) energy in column F and negative (pumped) energy in column E.

The user should note that daily energy balance from each storage asset should be zero, after accounting for round-trip efficiency (e.g. a pumped storage with a 75% round-trip efficiency will over the course of the day withdraw three quarters of the energy it injects in storage).

      - Imported & exported energy from interconnectors

Unconstrained interconnector flows are reported at various levels of temporal aggregation (by time period or on an annual basis) in ELSI. As noted in section 4.8 above, the modelling of interconnectors is different in GB-only and full NWE modes.

- In GB-only model, interconnectors to GB are handled like GB generators

  - In the O_Period_System results sheet, unconstrained GB interconnector flows (in MW) for each time period is written in the range rngOPGBUnConMO,

  - In the O_Annual_System results sheet, annual unconstrained generation (in TWh) for each GB interconnector is written in rows 6 to 57. The left hand side table calculates the annual energy generated, when the right hand side table evaluates the average annual load factor.

- In full NWE mode, GB & NWE interconnectors are handled as links

  - In the O_Period_Network sheet, unconstrained link flows by time period (in MW) are reported in the range rngOPUnConLinkFlows. Annual unconstrained interconnection flows (in TWh) are reported in column F (on the left hand side of the previous range) as well as in the O_Annual_System sheet (imports in cells G7 to G56 and exports in cells H7 to H56).

      - Final energy demand & loss of load (DSR)

Technically speaking, the energy demand reported by ELSI is constrained (i.e. not unconstrained), though the two values should be identical. Energy demands by price zone and time period (in MW) are written in the rngOPZoneDemand range. Annual energy demand values (in TWh) are in the H column. The annual demand figures are picked in the O_Annual_System sheet (cells F7 to F50).

Loss of load (named DSR in ELSI) is also reported for debugging purposes in the O_Period_System sheet. Unconstrained GB DSR volumes by time period (in MW) are reported in the range rngOPGBUnConDSR. Annual volumes (in TWh) are in the corresponding H column. Note that annual GB DSR volumes are picked up in the O_Annual_System sheet (cells BG7 to BG50) & the O_Multi_Detail (row 460 to 499) sheets.

Technically speaking, the NWE loss of load reported in ELSI is constrained (i.e. not unconstrained). Note that as there are no constraints in NWE markets, unconstrained & constrained NWE DSR should be identical. NWE DSR by time period (in MW) is reported in the rngOPEUDSR range in the O_Period_System sheet. Annual volumes (in TWh) are in the corresponding H column.

    - Unconstrained GB boundary flows

Unconstrained (or day-ahead) flows across GB transmission boundaries are calculated by ELSI and reported by boundary & time period (in MW) in the rngOPUnconLimFlow range in the O_Period_Network sheet. Annual unconstrained energy transfer (in TWh) is reported in the corresponding column F.

Quartiles & extreme values (min & max) of unconstrained GB boundary flows are reported in the O_Annual_Network sheet (cells G8 to K42), in the O_Annual Summation sheet (cells B242 to F276) & in the O_Multi_Detail sheet (rows 53 to 235).

    - Unconstrained zonal prices

ELSI reports unconstrained (or day-ahead) SMP (System Marginal Price) for each price zone & time period (in MW) in the rngOPUnConSMP range in the O_Period_System sheet. Annual volumes (in TWh) are in the corresponding H column. Note that, for an unconstrained run SMPs for all GB zones should be identical in a given time period. Time-weighted annual day-ahead prices are picked in the O_Annual_System sheet (cells D7 to D50) and demand weighted annual day-ahead prices in the O_Multi_Summary sheet (rows 243 to 253).

    - Day-ahead emissions

ELSI reports unconstrained day-ahead emissions for each price zone & time period (in tonnes of CO2) in the rngOPZoneEmissionsUnCon range in the O_Period_System sheet. Annual volumes (in Mtonnes) are in the corresponding H column. The annual emission figures are picked in the O_Annual_System sheet (cells E7 to E50) and in the O_Multi_Summary sheet (rows 243 to 253).

## Simulated re-dispatch results

    - Constrained & re-dispatched energy volumes

      - Generation from power generators

ELSI reports constrained generation from power generators for both GB (by transmission zone) & NWE markets at various levels of time aggregation:

- In the O_Period_System results sheet, constrained GB generation (in MW) (incl. proxy interconnectors) for each time period is written in the range rngOPGBConMO, and similarly detailed constrained NWE generation is written in the range rngOPEUConMO,

- In the O_Annual_System results sheet, annual constrained generation by plant type & zone is written in rows 61 to 112. The left hand side table calculates the annual energy generated, when the right hand side table evaluates the average annual load factor for each of these technologies in each of the markets considered,

- The O_Annual Summation, O_Multi_Results_Template, O_Multi_Detail & O_Multi_Summary results sheet all contain within-day generation by plant type for both:

  - GB: rows 15 to 62 in O_Annual Summation, rows 62 to 109 in O_Multi_Results_Template, rows 644 to 1218 in O_Multi_Detail & rows 68 to 115 in O_Multi_Summary, and

  - Other NWE markets: rows 65 to 174 in O_Annual Summation, rows 112 to 221 in O_Multi_Results_Template & rows 118 to 227 in O_Multi_Summary.

Re-dispatched volumes are calculated:

- At a time period level in the O_Period_System results sheet and reported in the range rngOPGBReSchedMO for GB generators. There is no equivalent for NWE generators as the cost of re-dispatching from NWE markets are borne solely by the interconnectors in ELSI,

- At an annual level, bids & offer volumes by GB plant types are reported in rows 15 to 62 of the O_Annual Summation sheet, and in the O_Multi_Detail sheet in rows 1221 to 1795 (for bid-off volumes), & rows 1798 to 2372 (for volumes offered on).

      - For storage assets

Storage assets can both inject (release) energy to and withdraw (pump) energy from the system. ELSI reports constrained generation from storage assets with other generators (as above) but it is also setup to report these quantities separately.

In the O_Period_System result sheet, (pumped) storage constrained generation (in MW) (both injection & withdrawal) is written in the range rngOPConPS. Corresponding constrained generation (in MWh) is calculated in rows 3983 to 4054 of the same sheet. It is used to separate annual generation between positive (released) energy in column F and negative (pumped) energy in column E.

The user should note that daily energy balance from each storage asset should be zero, after accounting for round-trip efficiency (e.g. a pumped storage with a 75% round-trip efficiency will over the course of the day withdraw three quarters of the energy it injects in storage).

Re-scheduled storage generation (in MWh) is calculated in rows to of the same O_Period_System result sheet in rows 4058 to 4129. These numbers are aggregated for all GB storage units in row 4130.

      - Imported & exported energy from interconnectors

Constrained interconnector flows are reported at various levels of temporal aggregation (time period or annual) in ELSI. As noted in section 4.8 above, the modelling of interconnectors is different in GB-only and full NWE modes.

- In GB-only model, interconnectors to GB are handled like GB generators

  - In the O_Period_System results sheet, constrained GB interconnector flows (in MW) for each time period is written in the range rngOPGBConsMO, re-scheduled energy flows are written in the rngOPGBReSchedMO of the same sheet,

  - In the O_Annual_System results sheet, annual constrained generation (in TWh) for each GB interconnector is written in rows 62 to 111. The left hand side table calculates the annual energy generated, when the right hand side table evaluates the average annual load factor.

- In full NWE mode, GB & NWE interconnectors are handled as links

  - In the O_Period_Network sheet, constrained link flows by time period (in MW) are reported in the range rngOPConsLinkFlows. An additional table (rows 72 to 89) in the O_Period_Network sheet calculates the constrained link flows in MWh. It is used to derive annual import & export volumes in sheet O_Annual_Summation (rows 5 to 12) and in sheet O_Multi_Summary (rows 50 to 65).

  - Annual constrained interconnection flows (in TWh) are reported in column F (on the left hand side of the previous ranges) as well as in the O_Annual_System sheet (imports in cells G62 to G111 and exports in cells H62 to H111).

  - Re-scheduled energy flows are reported in rows 104 to 130 of sheet O_Period_Network.

      - Final energy demand & loss of load (DSR)

Energy demands by price zone and time period (in MW) are written in the rngOPZoneDemand range. Annual energy demand values (in TWh) are in the H column. The annual demand figures are picked in the O_Annual_System sheet (cells F7 to F50).

Loss of load (named DSR in ELSI) is also reported for debugging purposes in the O_Period_System sheet. Constrained GB DSR volumes by time period (in MW) are reported in the range rngOPGBConsDSR. Annual volumes (in TWh) are in the corresponding H column. Note that annual GB DSR volumes are picked up in the O_Annual_System sheet (cells BG62 to BG111) & the O_Multi_Detail (row 502 to 541) sheets.

NWE DSR by time period (in MW) is reported in the rngOPEUDSR range in the O_Period_System sheet. Annual volumes (in TWh) are in the corresponding H column.

    - Balancing Mechanism costs

Balancing Mechanism (BM) costs can come from:

- GB generation re-dispatch,

- GB interconnector re-dispatch,

- GB storage re-dispatch, and

- DSR (loss of load) use.

The costs of these re-dispatching actions are assessed and accrued by plant category over the year modelled. Rows 3524 to 3528 in the O_Period_System sheet calculate the BM costs for all the above four categories. In more details:

- GB generation BM costs (including costs of re-dispatching GB interconnectors in GB-only mode) are written by plant type, zone & at time period granularity in the range rngOPGBBMCosts in the O_Period_System sheet. Annual aggregation of these costs (in £m) is provided in the corresponding H column.

- GB interconnector BM costs (in full NWE model) are written by interconnector & at time period granularity in rows 134 to 160 in the O_Period_Network sheet. Annual aggregation of these costs (in £m) is provided in the corresponding F column.

- GB storage BM costs are written by plant type, zone & at time period granularity in rows 4136 to 4207 in the O_Period_System sheet. Annual aggregation of these costs (in £m) is provided in the corresponding H column.

- GB DSR BM costs (offer only as it does not make sense for loss of load to be curtailed) are written by zone & at time period granularity in rows 4355 to 4394 in the O_Period_System sheet. Annual aggregation of these costs (in £m) is provided in the corresponding H column.

    - Constrained GB boundary results

      - Flows

Constrained flows across GB transmission boundaries are calculated by ELSI and reported by boundary & time period (in MW) in the rngOPConsLimFlow range in the O_Period_Network sheet. Annual constrained energy transfer (in TWh) is reported in the corresponding column F.

Quartiles & extreme values (min & max) of constrained GB boundary flows are reported in the O_Annual_Network sheet (cells G8 to K42), in the O_Annual Summation sheet (cells G242 to K276) & in the O_Multi_Detail sheet (rows 238 to 420).

      - Shadow prices & estimated constraint values

Shadow prices are a key indicator for constraints in the Linear Program (LP). They represent the marginal system benefit of relaxing the constraint they are associated with. In our case, GB transmission constraints limit the amount of energy that can flow across GB zones (in a constrained run only). If this constraint is binding i.e. the transmission line is operating at full nameplate rating, there is a value to the system in allowing for more transfer capacity. This value is expressed in the shadow price associated with the constraint (in £/MWh). As an example, a shadow price of £100/MWh in boundary B6 on January 1st, during the peak time periods means that at this time, allowing for an extra MWh to flow in B6 would lead to a system cost reduction of £100.

Shadow prices associated with GB transmission boundaries are reported on a time period granularity (in £/MWh) in range rngOPLimShadowP in the O_Period_Network sheet. Note that no aggregation on an annual level is provided as shadow prices will be zero whenever the boundary is not operating at full rating (i.e. the constraint is not binding).

Estimated constraint values, calculated as the product of re-dispatched flows times the shadow price, are reported in ELSI for a simplified estimate of constraint value. Note that this is just an indicative calculation and relies on several assumptions (e.g. shadow prices stay constant as re-dispatched volume increases, transmission constraints can be resolved separately for each boundary, etc.). It is reported for information in the O_Period_Network sheet (rows 317 to 351). Annual estimated constraint values are calculated in the corresponding column F.

      - Estimated Balancing Mechanism costs

On top of the estimated constraint costs, calculated from transmission constraint shadow prices as above, a simple apportioning of BM costs by constraint has been defined in ELSI on the basis of re-scheduled flows. Note that this too relies on very simple assumptions and should be viewed as an information indicator, not the correct apportioning of BM costs by constraint, which is out of the scope of this tool. Note also that some very high values can appear here where the total re-dispatched volume across all GB boundaries is very small, but made of large regional upward & downward re-scheduling e.g. when Scottish wind curtailed, and the energy replaced by more imports from the South of England.

These values are reported by time period in the O_Period_Network sheet (rows 355 to 389).

    - Constrained zonal prices

ELSI reports constrained (or within-day) SMP (System Marginal Price) for each price zone & time period (in MW) in the rngOPConsSMP range in the O_Period_System sheet. Annual volumes (in TWh) are in the corresponding H column.

Time-weighted annual within-day prices are picked in the O_Annual_System sheet (cells D62 to D111) and in O_Multi_Detail (rows 11 to 50).

    - Within-day emissions

ELSI reports constrained within-day emissions for each price zone & time period (in tonnes of CO2) in the rngOPZoneEmissionsCons range in the O_Period_System sheet. Annual volumes (in Mtonnes) are in the corresponding H column. The annual emission figures are picked in the O_Annual_System sheet (cells E62 to E111).

## Optimizer diagnostics

ELSI reports performance indicators from the LP solver used to find the optimal power dispatch. At a high level, the solver spends most of its time finding the best use of interconnectors & storage assets. In practice, this is done using a two-level nested loop (first network, then storage) to manage convergence.

    - Network iterations

ELSI iterates several dispatch options on NWE interconnectors to find the optimal dispatch. The number of network iterations is reported in range rngOPNetIter in sheet O_Period_System. The corresponding network error is reported in range rngOPNetErr in sheet O_Period_System.

    - Pump iterations

ELSI iterates several dispatch options on storage assets to find the optimal dispatch.

The number of storage iterations for phase 1 (i.e. unconstrained) is reported in range rngOPPumpIter1 in sheet O_Period_System. The corresponding storage error is reported in range rngOPPumpErr1 in sheet O_Period_System.

The number of storage iterations for phase 2 (i.e. constrained) is reported in range rngOPPumpIter in sheet O_Period_System. The corresponding storage error is reported in range rngOPPumpErr in sheet O_Period_System.

- Geographical scope covered in ELSI

  - Map of North West European markets represented in ELSI

Figure 42	Map of North West European markets represented in ELSI

ZZ

GB

DK

DE

BE

NL

NO

IC

ES

IE

FR

  - Map of the GB transmission system (ETYS 2016)

Figure 43	Map of the GB transmission system (ETYS 2016)

  - ELSI zones

The GB system flop zones are mapped onto ELSI zones according to Table 2 below.

Table 2	Mapping of ELSI zones to GB system FLOP zones

| ELSI zone name | ELSI zone code | System zones |
| --- | --- | --- |
| London | A | A |
| Bramley | B | B4 |
| Thames Estuary | C | C1, 2, 3, 5 & 6 |
| Cleve Hill | C1 | N/A |
| North of London | D | D4 & 5 |
| North Somerset | E | E7 |
| Devon & Cornwall | F | F6 |
| West Central | G | G1, 5, 6 & 7 |
| South Wales | H | H1, 2 & 6 |
| SPT South | I | S1, 2, 3, 4, 8 & 9 |
| East Anglia | J | J4, 6, 7 & 8 |
| Trent | K | K1, 2, 4, 5 & 6 |
| West Midlands | L | L1, 2, 3, 5, 7 & 8 |
| Deeside & Legacy | M | M4 & 5 |
| North West | N | N1, 2, 3, 4, 5, 6, 7, 8 |
| SPT North | O | S5, 6 & 7 |
| Yorkshire | P | P1, 2, 3, 4, 5, 6 & 8 |
| North-West Borders | Q | Q8 |
| Cumbria & Lancashire | R | R4, 5 & 6 |
| Central South Coast | S | B1 |
| Central South Coast 2 | S1 | B2 & 3 |
| Southern Kent and East Sussex | T | C4, 7 & 9 |
| Norwich, Bramford & Sizewell | U | J2, 3 & 5 |
| Ffestiniog | V | M7 |
| Angus, Perth and Kinross | W | T4 |
| Argyll and Bute | X | T3 |
| Aberdeenshire | Y | T2 (split) |
| Aberdeenshire 2 | Y1 | T2 (split) |
| Southern Highlands | Z | T1 (split) |
| Southern Highlands 2 | Z1 | T1 (split) |
| Western South Coast | 1 | E1, 6 & 8 |
| Walpole | 2 | J1 |
| Irish Onshore/Offshore Wind – Pentir | 3 | N/A |
| North-West Wales | 4 | M6 |
| Anglesey | 5 | M8 |
| Humberside | 6 | P7 |
| North East | 7 | Q2, 4, 5, 6 & 7 |
| Northern Highlands | 8 | T5 |
| Irish Onshore Wind – Pembroke | 9 | N/A |
| Oxfordshire | 0 | D6 |

- Reference data sources

  - UK Future Energy Scenarios (2016/17)

Government and EU targets require that by 2020, 15% of all energy will be from renewable sources. A series of four carbon budgets set by the Committee on Climate Change (CCC) and subsequently adopted by the Government, call for carbon emissions to be reduced by 37% compared to 1990 levels by 2020 and 60% by 2030. National Grid developed 3 different scenarios that examine the potentials of achieving these targets.

The Gone Green scenario produces projections at a UK level. This scenario anticipates that both the renewable energy and carbon emissions targets will be achieved. This is the most developed scenario out of the three and is believed to be the most likely possibility.

Slow Progression is modelled at a GB level rather than a UK. This scenario is not developed to the same detail as the Gone Green scenario but indicative calculations show that the 2020 emissions target will not be reached until 2026.

The Accelerated Growth scenario assumes that offshore generation builds up far more quickly than in the other scenarios. The renewable and carbon emission targets are met earlier in Accelerated Growth than in Gone Green.

Further information regarding the detailed assumptions underlying each of the scenarios can be found within the UK Future Energy Scenarios document published by National Grid at (http://www.nationalgrid.com/NR/rdonlyres/86C815F5-0EAD-46B5-A580-A0A516562B3E/50819/10312_1_NG_Futureenergyscenarios_WEB1.pdf)

  - BID3 model

Since 2016/17, the BID3 has replaced ELSI as the preferred model for producing the Network Options Assessment. The SO has published a detailed functional report explaining the reasons for this change and describing the main functions of BID3 here: http://www2.nationalgrid.com/WorkArea/DownloadAsset.aspx?id=8589938715.

  - Detailed data sourcing

Data from FES & BID3 was used to prepare the data release in the public version of ELSI. Table 3 below recaps the sourcing for each key data item.

Table 3	Data sources in public ELSI

| Element | Sheets | Source |
| --- | --- | --- |
| GB generation mix | I_GB_Capacity_xx | BID3 (aggregated) |
| NWE generation mix | I_NWE_Capacity | ENTSO-E (aggregated) |
| Power plant operational parameters | I_Plant_Assumptions | BID3 (aggregated) |
| Commodity prices | I_Commodities | FES, except BID3 for transport adders |
| Power plant bid/offer scalars | I_GB_Bid_Offer | Historical ELSI values |
| ROC price projections | I_GB_Bid_Offer | BID3 |
| GB peak demands by zone | I_Peak_Demands | BID3 |
| NWE peak demands | I_Peak_Demands | BID3 (includes effect of non-modelled markets) |
| GB load duration curve | I_Load_Duration | BID3 (includes effect of non-modelled markets) |
| NWE load duration curve | I_Load_Duration | BID3 (includes effect of non-modelled markets) |
| GB wind load factors | I_Wind | BID3 (aggregated) |
| NWE wind load factors | I_Wind | BID3 (aggregated) |
| GB solar PV load factors | I_Solar | BID3 (aggregated) |
| NWE solar PV load factors | I_Solar | BID3 (aggregated) |
| GB network topology | I_Network_Static | BID3 (from National Grid) |
| GB network capabilities | I_Network_Capabilities | National Grid as BID3 data is Relevant System Planning Information |
| GB network outages | I_Network_Outages | BID3 |
| NWE interconnectors | I_Network_Links | BID3 (aggregated) |
| Hydro run-of-river input | Optimiser_System | ENTSO-E (historical NWE average) |
| Storage efficiency | Optimiser_System | BID3 (aggregated) |

The NOA report and associated documents can be downloaded at: http://www2.nationalgrid.com/UK/Industry-information/Future-of-Energy/Network-Options-Assessment/

Outputs from this updated ELSI model (using a different data set) were compared with the ones from NOA’16.

Use of ELSI and related tools is conditional on accepting the terms of the disclaimer set forth by National Grid SO. Please read carefully the notes provided with the model.

See section 3.2.1 for further details on how this can be done in practice.

The 2016 FES report can be downloaded from: http://fes.nationalgrid.com/fes-document/

See section 4.1.2 for more details on how to do this in practice.

The report is available here: http://www2.nationalgrid.com/WorkArea/DownloadAsset.aspx?id=8589938716

In this dispatch configuration, each NWE zone holds an import and export price which can be changed by the user.

Note that on top of the 9 legitimate NWE markets modelled in ELSI, we have added a ‘dummy’ ZZ market zone as ELSI is not able to run with only one market (this would otherwise be the case in the GB-only mode). The ZZ market is configured with a very small demand, a small reservoir hydro generator (as one is required for the reservoir hydro heuristic to work and GB does not have any in the present setup), a small pumped storage generator & a small ‘Not identified’ generator which will drive the price in the ZZ market.

Note that these zones do not correspond to either minor or major FLOP zones.

In practice this means ELSI optimizes the dispatch of each day taken individually in turn e.g. Jan. 1st, then Jan. 2nd, etc.

To abide by RSPI (Relevant System Planning Information) rules regarding sharing of sensitive SO data, GB peak annual electricity demands have been aggregated by ELSI zone. Nonetheless, as mentioned earlier, the tool leaves the users free to populate this data at FLOP zone level if they wish.

Due to averaging of hours in time periods, the load value reported never reaches 100%.

These are modelled on the same basis as the Initial National Demand Outturn (INDO).

Note that some power plant e.g. biomass, wind are modelled with a symbolic efficiency of 100% for simplicity.

In reality, power plant are in general more efficient when they run close to full nameplate capacity so SRMC tends to reduce with loading.

Note that VOM costs are not included in this version of ELSI.

In the current model load factors for enhanced wave generation are equated to the ones of offshore wind. Users are free to update this assumption as they see fit.

Users remain free to define other scenarios, as they see fit.

For instance, project B can only be active is project A is installed first, or the capability of project A & B together is less than the sum of the capabilities of project A & B.

The model is initially configured to full export (added to the peak demand values in the appropriate zones).

In practice, the NWE unconstrained market price is decreased to take into account losses on the interconnector in the dispatch decision.

In practice, the NWE unconstrained market price is increased to take into account losses on the interconnector in the dispatch decision.

The menu on the top left of the O_Period_System sheet allows for navigating straight to the desired output.

Since transmission constraint are not applied at this stage, power prices are identical across the whole market.

Note that for in the case of both GB & NWE markets, annual generation (in TWh) is calculated in column H to the left of the unconstrained generation range.

The User should note that during testing, we have observed that the value of water can start very high (value is equivalent to SRMC of OCGT) in the beginning of the year and then taper off progressively to the long-term average value of water. We have proposed values for the parameters controlling the reservoir hydro heuristic in order to minimize the time taken for the value of water to stabilize to long-run average.

Note that annual storage generation (in TWh) is calculated in column H to the left of the unconstrained generation range.

Note that energy demands can be recalculated by multiplying the load duration factor with the peak demand for the chosen zone & time period.

Note that for in the case of both GB & NWE markets, annual generation (in TWh) is calculated in column H to the left of the constrained generation range.

Note that this is further broken down by GB transmission zone (on top of power plant type).

Note that for in the case of both GB & NWE markets, annual generation (in TWh) is calculated in column H to the left of the constrained generation range.

Note that annual storage generation (in TWh) is calculated in column H to the left of the constrained generation range.

Note that this is unrelated to estimated BM costs as it does not take into account bids & offers from generators.

For information, the LP solver uses Benders decomposition.

Source: http://www2.nationalgrid.com/UK/Industry-information/Future-of-Energy/Electricity-ten-year-statement/